# Vivado Implementation Guide — MLP Accelerator on Zybo Z7-10

## Overview

Network architecture: 784 → FC1(64, ReLU) → FC2(32, ReLU) → FC3(10, argmax)

Quantization: INT8 symmetric post-training

Target board: Digilent Zybo Z7-10 (XC7Z010-1CLG400C)

PL clock: 100 MHz (FCLK_CLK0 from Zynq PS)

Inference time: ~1.58 ms per image

### File Map

mlp_accelerator_zybo_z710/
├── constraints/
│   └── zybo_z710_mlp.xdc        Timing and I/O constraints
├── hardware/
│   ├── rtl/
│   │   ├── mlp_top.v            Top-level wrapper; AXI-Lite + BRAM Port B ports
│   │   ├── mlp_engine.v         Serial MAC inference FSM
│   │   ├── axilite_ctrl.v       AXI4-Lite slave (CTRL / STATUS / RESULT registers)
│   │   └── mlp_params.vh        AUTO-GENERATED by train_and_export.py
│   └── firmware/
│       ├── mlp_bram_init.h      ARM C driver header
│       ├── mlp_bram_init.c      ARM C driver — bulk-loads BRAM at boot
│       └── weights_biases.h     AUTO-GENERATED by train_and_export.py
└── software/
    └── train_and_export.py      Training + quantization + header export

## System Architecture

Zynq PS (ARM Cortex-A9)
   │
   │  M_AXI_GP0 (32-bit AXI, 100 MHz)
   │
   ▼
AXI SmartConnect (1 master → 3 slaves)
   │
   ├──► AXI BRAM Controller 0  ──► BMG 0 Port A (32-bit, write path from PS)
   │                                BMG 0 Port B (8-bit, enb-controlled) ──► mlp_engine param_bram_*
   │                                [Param BRAM — 64 KB: INT8 weights + INT32 biases]
   │
   ├──► AXI BRAM Controller 1  ──► BMG 1 Port A (32-bit, write path from PS)
   │                                BMG 1 Port B (8-bit, enb-controlled) ──► mlp_engine input_bram_*
   │                                [Input BRAM — 1 KB: 784 INT8 pixel bytes]
   │
   └──► mlp_top AXI-Lite slave
            │
            ├── axilite_ctrl  (CTRL / STATUS / RESULT registers)
            └── mlp_engine    (serial MAC FSM — INT32 accumulator)

### Data Flow

Boot: ARM writes weights + biases to Param BRAM via AXI BRAM Ctrl 0 (mlp_bram_init())

Per inference:

a. ARM writes 784 pixel bytes to Input BRAM via AXI BRAM Ctrl 1

b. ARM writes 0x1 to CTRL register → triggers start pulse to engine

c. Engine reads weights byte-by-byte and biases 4 bytes (INT32, little-endian) via Port B; reads input pixels byte-by-byte via Port B

d. Engine computes FC1 → ReLU → FC2 → ReLU → FC3 → argmax

e. Engine asserts done pulse, writes 4-bit class to RESULT register

f. ARM polls STATUS[0] (done), then reads RESULT[3:0]

### Address Map

| Peripheral | Base Address | Size |
| --- | --- | --- |
| Param BRAM (AXI BRAM Ctrl 0) | 0x4000_0000 | 64 KB |
| Input BRAM (AXI BRAM Ctrl 1) | 0x4001_0000 | 4 KB |
| mlp_top AXI-Lite | 0x4002_0000 | 4 KB |

### AXI-Lite Register Map

| Offset | Name | Dir | Bits | Description |
| --- | --- | --- | --- | --- |
| +0x00 | CTRL | W | [0] | Write 1 to start inference (auto-clears) |
| +0x04 | STATUS | R | [0] | Done flag (latched, clears on read) |
| +0x04 | STATUS | R | [1] | Busy flag (high while engine is running) |
| +0x08 | RESULT | R | [3:0] | Predicted class 0–9 |

### Param BRAM Byte-Address Layout

| Byte Offset | Contents | Size |
| --- | --- | --- |
| 0x0000_0000 | fc1_weights [64×784 INT8] | 50 176 B |
| 0x0000_C400 | fc1_bias [64 INT32] | 256 B |
| 0x0000_C500 | fc2_weights [32×64 INT8] | 2 048 B |
| 0x0000_CD00 | fc2_bias [32 INT32] | 128 B |
| 0x0000_CD80 | fc3_weights [10×32 INT8] | 320 B |
| 0x0000_CEC0 | fc3_bias [10 INT32] | 40 B |
| Total |  | 52 968 B (~51.7 KB) |

## Part 1 — Create the Vivado Project

Open Vivado → Create Project → RTL Project

Uncheck *Do not specify sources at this time*

Add Sources — add:

hardware/rtl/mlp_top.v

hardware/rtl/mlp_engine.v

hardware/rtl/axilite_ctrl.v

hardware/rtl/mlp_params.vh *(add as a header/include source, not a top module)*

Add Constraints — add constraints/zybo_z710_mlp.xdc

Default Part — search xc7z010clg400-1 and select

*(or select board "Zybo Z7-10" if Digilent board files are installed)*

## Part 2 — Package mlp_top as a Custom IP

The block design requires mlp_top to be packaged so Vivado can infer its AXI-Lite interface.

Tools → Create and Package New IP

Choose Package a specified directory → point to the hardware/rtl/ folder

Set name: mlp_accelerator, version: 1.0

In the IP Packager:

Ports and Interfaces tab: Vivado should auto-infer the S_AXI (AXI4-Lite) interface from the s_axi_* port names. If not, right-click → Add Interface → AXI4-Lite → assign ports manually.

The param_bram_* and input_bram_* ports are left as plain ports; they connect directly to BMG Port B nets in the block design.

Review and Package → Package IP

## Part 3 — Build the Block Diagram

Create Block Design: Flow Navigator → IP Integrator → Create Block Design

Name: mlp_system

Add and configure each IP block as described below.

### 3.1 — Zynq Processing System (PS7)

Add IP: ZYNQ7 Processing System

Run Block Automation → accept defaults, then double-click to configure:

| Tab | Setting | Value |
| --- | --- | --- |
| PS-PL Configuration → AXI Non-Secure Enablement | GP0 AXI Master Interface | ✓ Enabled |
| Clock Configuration → PL Fabric Clocks | FCLK_CLK0 Requested Frequency | 100 MHz |
| PS-PL Configuration → General → Enable Clock Resets | FCLK_RESET0_N | ✓ Enabled |

Exposed ports: FCLK_CLK0, FCLK_RESET0_N, M_AXI_GP0

### 3.2 — Processor System Reset

Add IP: Processor System Reset

| Port | Connect to |
| --- | --- |
| slowest_sync_clk | FCLK_CLK0 |
| ext_reset_in | FCLK_RESET0_N |
| dcm_locked | Tie to 1'b1 (no external MMCM) |

Output used: peripheral_aresetn[0:0] (active-low reset for all AXI peripherals)

### 3.3 — AXI SmartConnect

Add IP: AXI SmartConnect

| Setting | Value |
| --- | --- |
| Number of Slave Interfaces | 1 |
| Number of Master Interfaces | 3 |
| Clock | FCLK_CLK0 |
| Reset | peripheral_aresetn |

Connections:

S00_AXI ← M_AXI_GP0 (from Zynq PS)

M00_AXI → AXI BRAM Controller 0

M01_AXI → AXI BRAM Controller 1

M02_AXI → mlp_top AXI-Lite

### 3.4 — AXI BRAM Controller 0 (Param BRAM)

Add IP: AXI BRAM Controller

| Setting | Value |
| --- | --- |
| AXI Protocol | AXI4 |
| Data Width | 32 |
| Single Port BRAM | ✓ (Port A only faces AXI) |

| Port | Connection |
| --- | --- |
| s_axi_aclk | FCLK_CLK0 |
| s_axi_aresetn | peripheral_aresetn |
| s_axi_* | SmartConnect M00_AXI |
| BRAM_PORTA | BMG 0 Port A (see 3.6) |

### 3.5 — AXI BRAM Controller 1 (Input BRAM)

Add IP: AXI BRAM Controller — identical configuration to 3.4.

| Port | Connection |
| --- | --- |
| s_axi_* | SmartConnect M01_AXI |
| BRAM_PORTA | BMG 1 Port A (see 3.7) |

### 3.6 — Block Memory Generator 0 (Param BRAM — 64 KB)

Add IP: Block Memory Generator

#### Port A (32-bit — AXI side)

| Setting | Value |
| --- | --- |
| Interface Type | Native |
| Memory Type | True Dual Port RAM |
| Write Width | 32 |
| Write Depth | 16 384 (= 65 536 bytes ÷ 4) |
| Read Width | 32 |
| Operating Mode | Read First |
| Primitives Output Register | Unchecked (DO_REG=0 → 1-cycle latency) |
| Enable Port Type | Always Enabled |

#### Port B (8-bit — engine side)

| Setting | Value |
| --- | --- |
| Write Width | 8 |
| Read Width | 8 |
| Write Depth | 65 536 (auto-filled from asymmetric ratio) |
| Operating Mode | Read First |
| Primitives Output Register | Unchecked (DO_REG=0 → 1-cycle read latency) |
| Enable Port Type | Use ENB Pin |

Why Use ENB Pin: mlp_engine drives param_bram_en and only asserts it

when it presents a valid read address. Selecting "Use ENB Pin" connects this

signal directly to the BRAM's enb port, so the BRAM only latches a new

address when the engine requests it. "Always Enabled" would expose no enb

port and cause the BRAM to continuously track the address bus — functionally

it still works but the param_bram_en output from the engine would be

unconnected and the design intent is lost.

#### Connections

| BMG 0 signal | Connects to |
| --- | --- |
| Port A (clka, addra, dina, douta, ena, wea, rsta) | AXI BRAM Ctrl 0 BRAM_PORTA |
| clkb | FCLK_CLK0 |
| addrb | param_bram_addr (from mlp_top) |
| enb | param_bram_en (from mlp_top) |
| web | Tie low (1'b0) |
| rstb | Tie low (1'b0) |
| doutb | param_bram_rdata → mlp_top input |
| dinb | Not connected (read-only port) |

### 3.7 — Block Memory Generator 1 (Input BRAM — 1 KB)

Add IP: Block Memory Generator — same type as BMG 0, different depth:

#### Port A

| Setting | Value |
| --- | --- |
| Write Width | 32 |
| Write Depth | 256 (= 1 024 bytes ÷ 4) |
| Read Width | 32 |
| Primitives Output Register | Unchecked |

#### Port B

| Setting | Value |
| --- | --- |
| Write Width | 8 |
| Read Width | 8 |
| Write Depth | 1 024 (auto) |
| Primitives Output Register | Unchecked (DO_REG=0 → 1-cycle read latency) |
| Enable Port Type | Use ENB Pin |

#### Connections

| BMG 1 signal | Connects to |
| --- | --- |
| Port A | AXI BRAM Ctrl 1 BRAM_PORTA |
| clkb | FCLK_CLK0 |
| addrb | input_bram_addr (from mlp_top) |
| enb | input_bram_en (from mlp_top) |
| web | Tie low (1'b0) |
| rstb | Tie low (1'b0) |
| doutb | input_bram_rdata → mlp_top input |
| dinb | Not connected (read-only port) |

### 3.8 — MLP Accelerator IP (mlp_top)

Add IP: mlp_accelerator v1.0 (your packaged IP from Part 2)

| Port | Connection |
| --- | --- |
| s_axi_aclk | FCLK_CLK0 |
| s_axi_aresetn | peripheral_aresetn |
| s_axi_* | SmartConnect M02_AXI |
| param_bram_* | BMG 0 Port B signals (see 3.6) |
| input_bram_* | BMG 1 Port B signals (see 3.7) |

## Part 4 — Assign Addresses

Open Window → Address Editor and set:

| Slave | Base Address | Range |
| --- | --- | --- |
| AXI BRAM Controller 0 (/axi_bram_ctrl_0/S_AXI) | 0x4000_0000 | 64K |
| AXI BRAM Controller 1 (/axi_bram_ctrl_1/S_AXI) | 0x4001_0000 | 4K |
| mlp_top (/mlp_accelerator_0/S_AXI) | 0x4002_0000 | 4K |

Important: MLP_BRAM_BASE_ADDR in firmware/mlp_bram_init.h must equal the

address assigned to AXI BRAM Controller 0 (0x40000000).

The ARM C driver uses 0x40010000 for the Input BRAM and 0x40020000 for the

AXI-Lite registers; update these if you use different addresses.

## Part 5 — Validate, Wrap, and Implement

Validate Design — press F6 (or right-click canvas → Validate Design).

Fix any red critical warnings before proceeding.

Generate HDL Wrapper — in Sources panel, right-click mlp_system →

Create HDL Wrapper → Let Vivado manage wrapper and auto-update → OK.

Set mlp_system_wrapper as Top (right-click → Set as Top).

Run Synthesis — Flow Navigator → Run Synthesis.

After synthesis, check the Utilization Summary:

| Resource | Expected Usage | Z7-10 Available |
| --- | --- | --- |
| BRAM 36K | ~15 | 60 |
| DSP48 | 0–2 | 80 |
| LUT | ~2 000–4 000 | 17 600 |
| FF | ~1 000–2 000 | 35 200 |

Run Implementation — Flow Navigator → Run Implementation.

Generate Bitstream — Flow Navigator → Generate Bitstream.

## Part 6 — Vitis / SDK Project

### 6.1 Export Hardware

File → Export Hardware → Include Bitstream → export .xsa file.

### 6.2 Create Platform and Application

Open Vitis → Create Platform Project from the .xsa file.

OS: standalone, Processor: ps7_cortexa9_0

Create a New Application Project → Empty Application (C)

### 6.3 Add Firmware Sources

Add to the application's src/ directory:

hardware/firmware/mlp_bram_init.h

hardware/firmware/mlp_bram_init.c

hardware/firmware/weights_biases.h *(generated by software/train_and_export.py)*

### 6.4 main.c Template

Critical: The Input BRAM must receive INT8 normalized pixels, not raw

uint8 values.  The model was trained on inputs normalized by

(pixel/255 − 0.1307) / 0.3081.  Feeding raw pixels to weights calibrated

for that range causes the FC1 accumulator to overflow by ~500×, collapsing

accuracy to ~10% (random).  The normalization step below fixes this.

#include <stdint.h>
#include <math.h>
#include "xil_io.h"
#include "xil_printf.h"
#include "mlp_bram_init.h"
#include "weights_biases.h"   /* provides MLP_INPUT_MEAN, MLP_INPUT_STD, MLP_INPUT_SCALE */

/* AXI-Lite register addresses — must match Vivado Address Editor */
#define MLP_AXILITE_BASE  0x40020000UL
#define MLP_CTRL_ADDR    (MLP_AXILITE_BASE + 0x00UL)
#define MLP_STATUS_ADDR  (MLP_AXILITE_BASE + 0x04UL)
#define MLP_RESULT_ADDR  (MLP_AXILITE_BASE + 0x08UL)

/* Input BRAM base address (AXI BRAM Controller 1) */
#define INPUT_BRAM_BASE  0x40010000UL

/* Clamp x to [lo, hi] */
static inline int clamp_int(int x, int lo, int hi) {
    return x < lo ? lo : (x > hi ? hi : x);
}

/*
 * normalize_pixels()
 * Convert 784 raw uint8 pixels to INT8 quantized values.
 *   1. Normalize:  x_f = (pixel/255.0 - MEAN) / STD
 *   2. Quantize:   x_i = round(x_f / INPUT_SCALE), clipped to [-128, 127]
 *   3. Write as two's-complement bytes to the Input BRAM.
 *
 * The RTL sign-extends each byte when reading Port B, so negative values
 * are handled correctly in hardware.
 */
static void normalize_pixels(const uint8_t *pixels, uint32_t n)
{
    volatile uint8_t *bram = (volatile uint8_t *)INPUT_BRAM_BASE;
    for (uint32_t i = 0; i < n; i++) {
        float x_f = ((float)pixels[i] / 255.0f - MLP_INPUT_MEAN) / MLP_INPUT_STD;
        int   x_i = (int)roundf(x_f / MLP_INPUT_SCALE);
        bram[i]   = (uint8_t)(int8_t)clamp_int(x_i, -128, 127);
    }
}

int main(void) {

    /* ── Step 1: Load weights/biases into Param BRAM (once at boot) ── */
    xil_printf("Loading MLP parameters into BRAM...\r\n");
    mlp_bram_init();
    if (mlp_bram_readback_check() != 0) {
        xil_printf("ERROR: BRAM readback failed. Check MLP_BRAM_BASE_ADDR.\r\n");
        return -1;
    }
    xil_printf("BRAM loaded OK.\r\n");

    /* ── Step 2: Normalize and write 784 pixels to Input BRAM ────── */
    extern uint8_t my_image[784];   /* raw uint8 pixels, 0–255 */
    normalize_pixels(my_image, 784);

    /* ── Step 3: Trigger inference ────────────────────────────────── */
    Xil_Out32(MLP_CTRL_ADDR, 0x00000001UL);

    /* ── Step 4: Poll STATUS[0] (done flag, read-to-clear) ───────── */
    while ((Xil_In32(MLP_STATUS_ADDR) & 0x1U) == 0U)
        ;

    /* ── Step 5: Read predicted class ────────────────────────────── */
    uint32_t cls = Xil_In32(MLP_RESULT_ADDR) & 0xFU;
    xil_printf("Predicted class: %d\r\n", (int)cls);

    return 0;
}

## Part 7 — Resource and Timing Summary

| Metric | Value |
| --- | --- |
| Inference time @ 100 MHz | ~1.59 ms |
| FC1 cycles: 64 × (12 bias + 784×3 MAC) | ~151 296 |
| FC2 cycles: 32 × (12 bias + 64×3 MAC) | ~6 528 |
| FC3 cycles: 10 × (12 bias + 32×3 MAC) | ~1 080 |
| Bias read overhead | 4 bytes × 3 cycles = 12 cycles/neuron (INT32, little-endian) |
| Param BRAM capacity needed | ~52.0 KB |
| BRAM blocks used (Param) | ~13 × RAMB36 |
| BRAM blocks used (Input) | 1 × RAMB36 |
| DSPs used | 1 (INT8 × INT8 multiplier) |
| Weight data type | INT8 (4 packed per 32-bit BRAM word) |
| Bias data type | INT32, signed (1 per 32-bit BRAM word) |
| MAC accumulator | INT32 |
| Inter-layer requantization | act_int8 = clip(ReLU(acc >> k), 0, 127) — shifts in mlp_params.vh |
| Inter-layer activations | ReLU + saturating clip to [0, 127] |

## Troubleshooting Checklist

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| mlp_bram_readback_check() returns mismatches | Wrong MLP_BRAM_BASE_ADDR | Match to Vivado Address Editor |
| STATUS never shows done | Engine start pulse not reaching PL, or bitstream mismatch | Re-export hardware, rebuild Vitis project |
| Wrong inference result | Input pixels not written to correct BRAM address | Verify INPUT_BRAM_BASE = address of AXI BRAM Ctrl 1 |
| Synthesis fails on mlp_engine.v | Vivado version < 2019.2 | Update or remove integer loop variable in reset block |
| BMG Port B data always 0 | DO_REG mismatch or EN not asserted | Confirm BMG config: Primitives Output Register = unchecked, Enable Port Type = Always Enabled |
| Timing not met | Clock too fast | Reduce FCLK_CLK0 to 50 MHz (adjust XDC constraint) |

## Regenerating Weights After Retraining

cd software/
python train_and_export.py
# Outputs:
#   hardware/firmware/weights_biases.h  — rebuild Vitis project to pick up new values
#   hardware/rtl/mlp_params.vh          — rebuild Vivado project to pick up new shifts
#   coe_files/                          — for Vivado BRAM initialisation (if needed)

Two things change after retraining:

Vitis (C firmware): Vitis recompiles mlp_bram_init.c automatically on the next build because it includes weights_biases.h. New weights, INT32 biases, input scale, and shift constants are picked up without any manual edits.

Vivado (RTL): rtl/mlp_params.vh is updated with the new MLP_REQ_SHIFT_FC1 and MLP_REQ_SHIFT_FC2 values. Re-synthesise and re-implement the Vivado project (Synthesis → Implementation → Generate Bitstream) to burn the new shifts into the PL.

Why the shifts matter: After each FC layer MAC, the INT32 accumulator is ≈10 000–50 000× larger than the float equivalent because both weights and inputs are quantised to small INT8 values. Without a right-shift before the ReLU clip, every positive neuron outputs 127 (binary activation), collapsing accuracy from ~97% to ~27%. The right-shift k = round(log2(act_scale / (w_scale × x_scale))) is computed from calibrated activation statistics and varies with each training run.